SURFACIAL MODIFICATION OF Ti3C2Tx MXENE USING CARBON ION IMPLANTATION FOR HYDROGEN GENERATION AND SUPERCAPACITOR APPLICATIONS

1Naresh Aneja, 1Akshidha Singla and  1* Aman Mahajan

1Department of Physics, Guru Nanak Dev University, Amritsar- 143005

*Corresponding author e-mail: aman.phy@gndu.ac.in

ABSTRACT

Hydrogen evolution reaction (HER) for energy conversion and supercapacitors (SCs) for consequent storage have attracted significant research interest for sustainable utilization of green and clean energy. Despite their efficient performance, still suffer from limitations which necessitates designing of proficient electrode/interfacial materials which can improve performance, stability and recyclability of devices. To address these issues, two-dimensional (2D) MXenes based materials are gaining interest by virtue of large surface area, high conductivity and tunable surface chemistry. However, the interlayer restacking issue directly hampers their scalability. To this, herein, surfacial modification of Ti3C2Tx MXene have been done using carbon ion implantation technique to efficiently increase the surfacial active sites. Finally, by employing a multifaceted approach, the morphology and structure of pristine and ion implanted MXene have been analyzed using X-ray diffraction (XRD) and field emission scanning electron microscopy (FESEM). Subsequently, the electrochemical investigation towards HER and SCs have been done using linear sweep voltammetry (LSV), cyclic voltammetry (CV), Galvanostatic charge-discharge (GCD) and electrochemical impedance spectroscopy (EIS). The ion implanted electrode exhibit reduced overpotential and Tafel slope with minimal charge transfer resistance and high capacitance value making them suitable for designing efficient MXene based electrochemical energy conversion and storage devices.

Keywords: Hydrogen Evolution Reaction, Supercapacitor, MXene, Carbon Ion Implantation