# OPTICALLY TUNABLE BROADBAND TERAHERTZ METAMATERIAL ABSORBER USING PHOTOCONDUCTIVE GALLIUM ARSENIDE

1* Medha Devgan, 2 Roopkiranpreet Kaur, 1 Sukhdeep Kaur, 3 Gagan Kumar

1 Department of Physics, Guru Nanak Dev University, Amritsar, Punjab-143005, India

2 Department of Physics, S. L. Bawa DAV College, Batala, Punjab – 143505, India

3 Department of Physics, Indian Institute of Technology Ropar, Punjab-140001, India

*Corresponding author e-mail address: devganmedha@gmail.com

# ABSTRACT

In this paper, we propose and numerically investigate a broadband terahertz (THz) metamaterial absorber operating in the 0.4-1.2 THz frequency range in which a photoconductive film of gallium arsenide (GaAs) is filled in to achieve optical tunability. GaAs is selected due to its strong photo-induced carrier generation property and well-researched ultrafast photoconductive response in the THz regime. In the absence of optical excitation, the structure exhibits near-zero absorption due to impedance mismatch. Upon optical pumping, free carriers are generated in the GaAs film. As the pump intensity increases, photogenerated carrier density rises proportionally, increasing the conductivity and progressively enhancing electromagnetic absorption. This mechanism enables tunable absorption ranging from approximately 0% in the dark state to about 90% under optimized optical pumping conditions. The proposed approach demonstrates that photoconductive GaAs can serve as an optically controllable resistive element in the THz regime, providing a compact, fabrication-friendly, and dynamically reconfigurable platform for THz modulators, adaptive absorbers and metasurfaces.

Keywords: Terahertz metamaterial, broadband absorption, electrical conductivity, semi-conductor